FAQ / Application Note

| Subject: Use the command capsule to override secure flash policy and program the BIOS image |  |
| --- | --- |
| FAQ Document No: S22012 | Date: 2022/06/24 |
| Model Name. Elkhart Lake models | Rev. A1 |
| Category: □General □H/W ■S/W □Others, BIOS |  |
| Purpose: Capsule mode is commonly used for firmware update methods when there’s a security BIOS concern and the common rule for programming BIOS is not visible. |  |

Because of security BIOS, the following message shows that the error occurs while in BIOS update.

Please add the parameter /capsule following the BIOS file, CEHL012.BIN, for example.

Press “Y” to proceed for capsule mode

The plaform key has been disabled, so the BIOS update can continue to go on

Press “Yes, I’m sure” to update the image

Press” Proceed with flash update”

BIOS firmware flash process is ongoing.

Press any key to reset the system after the update is completed.

Note that it’s sugegsted to use the afu BIOS flash tool with the version v5.15 or above