## BMC

| Update Method | OS | Tool |
| --- | --- | --- |
| Local update | Windows11 environment | Yafuflash.exe |
| Remote update | IPMI Web UI | No tool required |
|  | IPMI command | Yafuflash.exe |

Please refer readme for tool detail information.

### BMC update in WinPE environment

1. Copy update tool and BMC file to WinPE disk.

2. Plug the Windows10 disk to the Server and power on. When you hear BIOS ready beep, press F11 to enter boot menu and select the WinPE disk to boot.

3. Run the command with administrator privilege, Click “revocery.bat” and and Enter option “y” or follow the following steps.

After switch to the Win64 folder, Enter “Yafuflash -kcs HPM-ERSUA_v1.ima_enc” and Enter option “y”

Please wait. This may take few minutes

4. When the update process finishs, BMC will reset.

5. After BMC reset, enter “Yafuflash -kcs -mi” check BMC firmware version.

### BMC update using Web UI

1. Open web browser. Enter BMC IP address and log in. The default user name and password are admin/admin.

If you get a message that says “Your connection is not private”, just skip it.

Note: BMC IP address can be configured at BIOS menu.

2. Click the Maintenance tab, then Preserve Configuration.

Check all and Save.

3. Click the link to go to Firmware Upate.

4. Choose File to select BMC file.

5. Click the Start firmware update button, then scroll down and click Proceed.

The message appears, “Are you sure you want to flash?”. Click OK.

6. The message appears, “Firmware reset has been called. Close this current session, and open a new session after a copule of minutes.”. Click OK.

7. Reboot the server and then login to check the BMC firmware version.

### BMC update using IPMI tool

- Save BMC file to Yafuflash folder.

- Open Command Prompt (admin) and change directory to Yafuflash tool folder.

- Input the command:

Yafuflash.exe -nw -ip [BMC IP address] -U [user name] -P [user password] -pc -spi [BMC file name]. The default username and password are admin/admin.

Note: BMC IP address can be configured at BIOS menu.

4. When the following screen shows, please wait few seconds.

The update process will start.

5. When the update process finishs, BMC will reset.

6. Reboot the server. Check BMC firmware version by following fommand.

Yafuflash.exe -nw -ip [BMC IP address] -U [user name] -P [user password] -mi